Technical Requirements & Functional Requirements

Functional Requirements

1.0: The system shall separate cans by quantity needed (4 cans per lane)

1.0.1: The system shall maintain a count of cans in each lane until the required quantity is reached.

1.1: The system shall validate cans before packing and determine pass or fail

1.1.1: The system shall validate that four cans of each required can color are present.

1.1.2: The system shall validate the total number of cans.

1.1.3: The system shall validate that only one color is present per lane.

1.1.4: The system shall assign a pass or fail label.

1.1.5: The system shall verify that the empty box is correctly positioned before loading.

1.2: On Pass — The system shall push cans into the variety-pack box

1.3: On Fail — The system shall trigger an alarm, and the system shall halt the conveyor belt

1.4: The system shall be configurable, with required colors and quantities able to be changed.

Technical Requirements

1.0: PLC will act as the primary controller for all gates, sensors, pneumatic, and alarming operations. Each lane will use a Double Gate System to control the release of cans (4 cans for each lane). The gates shall operate from the side of the lane, which will allow the PLC to separate the required quantity of cans while holding the remaining upstream cans.

1.0.1: Inductive Proximity Sensor will be placed downstream in each lane. Each sensor signal will be sent to PLC, which will keep track of count until 4 cans are counted in each lane. Once the correct configuration is reached, the PLC will close the back gate to prevent additional cans from being released and open the front gate so the right can configuration can move onto the staging area.

1.1: OpenCV software and ELP camera will validate cans and determine pass/fail.

1.1.0: Functions can include - cv2.VideoCapture(), cv2.imread(), cv2.cvtColor(), cv2.GaussianBlur(), cv2.medianBlur(), cv2.Canny(), cv2.HoughCircles(), cv2.rectangle(), cv2.putText(), cv2.circle(). These functions are subject to change based on live testing.

1.1.1: ELP camera captures images of cans and OpenCV will validate correct colors.

1.1.1.1: Functions might include - cv2.cvtColor(), cv2.inRange(), cv2.bitwise_and(). These functions are subject to change based on live testing.

1.1.2: ELP camera and OpenCV will validate the total number of cans.

1.1.2.1: Functions might include - cv2.findContours(), cv2.contourArea(), cv2.boundingRect(). These functions are subject to change based on live testing.

1.1.3: ELP camera and OpenCV will validate that there is only one-color present per lane.

1.1.3.1: Functions might include- cv2.inRange(), cv2.findContours(), cv2.mean(). These functions are subject to change based on live testing.

1.1.4: After the camera is done capturing the images, OpenCV will assign either a pass or fail label.

1.1.5: A box-present (trigger switch) sensor will verify that an empty variety-pack carton is correctly positioned before the loading mechanism can operate. The sensor result will be sent directly to the PLC.

1.2: If Pass, the Raspberry Pi will signal to the PLC that this criterion has been met, the PLC will then verify that the box-present sensors detects the box, and then enable the pneumatic pusher to push the 12-pack into the variety-pack box

1.3: If Fail, the Raspberry PI, will signal this to the PLC, which will trigger the alarm for yards Workers to be alerted and factory indicator light shines on the specific lane error occurred. Conveyor stops until manually started through the push of a button.