Design Phase: Software & Defect Requirements Design

Team 1 will be using an ELP High Speed Global Shutter Camera to monitor the variety-pack beer cans through video as they are placed into the boxes. The system will check the colors of the cans to help verify that the correct varieties are present before manual packing.

Image Capture

The ELP camera will be mounted on the end of the process. Having a side-down view of the cans allows every can to be visible to the camera at the same time within a single image.  A lighting system will be placed with the camera since high shutter speeds require more light and will allow better visibility for the camera when capturing an image. Lighting also enables image quality to be more consistent overall.

Computer Vision Processing

As the ELP camera is not a smart camera, such as the Keyence camera, the images are processed externally using a device like a Raspberry Pi, industrial PC, etc. Once the image has been captured, it will be processed and analyzed by the computer vision software. The vision system will examine the captured images of the cans to identify can colors.

Methods in our code include:

- Define color requirements, should be changeable for different combinations

- Convert image to HSV values

- Detect colors

- Determine pass or fail

- On fail: Trigger red light

- On pass: Trigger green light

Pass/Fail Decision

The OpenCV software will make the pass or fail decision by comparing the detected arrangement against the required arrangement.

A 12-case variety pack will pass the camera inspection only if all required criteria is met:

- Each can matches the required variety or flavor

- Variety-pack cans are placed in the correct positions (upright)  BASED ON EMAIL RESPONSE

- No unexpected variety-pack can types are detected

A 12-case variety pack will fail the camera inspection if any of the following occur:

- Variety pack cans are positioned incorrectly

- An incorrect variety pack can flavor is detected

Reject Signals

When a variety-pack has failed the camera inspection, the camera vision system will send a signal to a warning system. The warning system will then use a stack light to both visually and audibly alert Yards workers, so the incorrect SKU can be removed before packaging.