Technical Requirements & Functional Requirements

Functional Requirements

1.0: The system shall validate cans before packing and determine pass or fail

1.0.1: The system shall validate that four cans of each required can color are present.

1.0.2: The system shall validate that only one color is present per lane.

1.0.3: The system shall assign a pass or fail label.

1.1: On Pass — The system shall trigger a green light.

1.2: On Fail — The system shall trigger a red light.

1.3: The system shall be configurable, with required colors and quantities able to be changed.

Technical Requirements

1.0: Raspberry Pi will act as the decision-maker for the alarming operations. It will allow the cans to be verified by color.

1.1: OpenCV software and ELP camera will validate cans and determine pass/fail.

1.1.0: Functions can include - cv2.VideoCapture(), cv2.imread(), cv2.cvtColor(), cv2.GaussianBlur(), cv2.medianBlur(), cv2.Canny(), cv2.HoughCircles(), cv2.rectangle(), cv2.putText(), cv2.circle(). These functions are subject to change based on live testing.

1.1.1: ELP camera captures images of cans and OpenCV will validate correct colors.

1.1.1.1: Functions might include - cv2.cvtColor(), cv2.inRange(), cv2.bitwise(). These functions are subject to change based on live testing.

1.1.2: ELP camera and OpenCV will validate that there is only one-color present per lane.

1.1.3.1: Functions might include- cv2.inRange(), cv2.findContours(), cv2.mean(). These functions are subject to change based on live testing.

1.1.4: After the camera is done capturing the images, OpenCV will assign either a pass or fail label.

1.2: If Pass, the Raspberry Pi will maintain the stack light’s green color.

1.3: If Fail, the Raspberry Pi, will change the stack light’s color to red and trigger an alarm sound.